LABORATORY SAFETY

GENERAL SAFETY PRECAUTIONS:

- Approach laboratory work with maturity.  Never run, push, or engage in horseplay or practical jokes of any kind in the laboratory.

- Unauthorized experiments are prohibited.  Do only those experiments assigned by your teacher.

- Never work alone in a science laboratory, and never work without teacher supervision.

- Never eat, drink, or chew gum in a science laboratory.  Do not store food or beverages in a lab.

- Never sit on lab tables, even if a lab is not in progress.

- Wear protective eye goggles and aprons whenever performing experiments in the laboratory.

- Restrain loose clothing (e.g. sleeves, full-cut shirts, etc.), long hair, and dangling jewelry.

- Footwear should cover feet completely.  No open-toe shoes.

- Keep your apparatus and desk area clean.  Store items not in use such as books, purses, and backpacks out of the way in designated storage areas.

- Keep chemicals, glassware, and other equipment away from the edge of the lab table.

- Know the location and proper use of the first aid kit, fire blanket, fire extinguisher, safety shower, and eyewash fountain.

- Know your lab group.  Do any of your group members wear contacts or have a medical condition such as asthma or epilepsy?  Know what to do in the case of an emergency.

SAFE PROCEDURES:

- Do not touch chemicals with your hands unless directed to do so.  Wash your hands thoroughly before and after working in a science laboratory, and after spill cleanups.

- Do not hold your face directly over an open container of chemicals.  When observing the odor of a substance, fan the vapor toward you by sweeping your hand over the top of the container.

- Never reach over an exposed flame or place flammable materials near a flame.

- When heating a substance, never use a closed container.  Always point the open end of the container away from yourself and others.

- Never leave heat sources unattended.  Turn off equipment when not in use.

- Use tongs, test-tube holders, or potholders to handle hot laboratory equipment.  Allow ample time for glassware to cool before handling.  Remember, hot glass looks like cool glass.

- Never touch electrical equipment with wet hands or place in an area that may be wet.

- Never use broken or chipped glassware.  Be sure all glassware is clean before you use it.

- When diluting an acid, always add the acid to the water.  “Do as you oughtta, add the acid to the watta.”

- Follow the directions from your teacher for the disposal of all chemicals.

- Do not use the sink to discard matches, paper, or other solid or slightly soluble materials.

ACCIDENTS:

- Report any accident or injury, no matter how minor, to your teacher.

- If a chemical spills on your skin or clothing, wash it off immediately with plenty of water.

- If a chemical gets into your eyes or on your face, have someone help you to the eyewash fountain and wash with plenty of water.  Contacts need to be removed immediately.

- Clean up all spills immediately.  Do not pick up broken glassware with your bare hands.  Use a dustpan and brush and place broken glass in the designated container.

- Smother small fires with a towel.  Use the fire blanket to extinguish clothing fires.  Only use the fire extinguisher on larger fires that do not involve people.

- Use the safety shower to treat full-body chemical spills or fires.